Requirements for obtaining a Certificate of Continued Occupancy.

Mark Lime the CCO Officer will contact you to schedule the inspection.

- Fill out the application completely. Submit it with your check for $200.00 for single-family dwellings and/or commercial property. Cash, personal check or certified bank check made out to the Borough of Bloomingdale is all that is accepted.

- Smoke Alarms on each level including basement, also outside bedroom areas, and in laundry room over dryer.

- Ten-year sealed battery-powered single station smoke alarms shall be installed and shall be listed in accordance with ANSI/UL 217,       incorporated herein by reference. However, A/C-powered single or multiple-station smoke alarms installed as part of the original construction or rehabilitation project shall not be replaced with battery-powered smoke alarms. The effective date of this subsection shall be January 1, 2019.

- Any hardwired smoke alarms shall be replaced per manufacturers recommendation and shall not be over ten (10) years old. Alarms over ten (10) years shall be replaced.

  - NFPA 72 Section 14

- Smoke alarms SHALL NOT remain in service longer than ten (10) years from date of manufacture, unless otherwise provided by the manufacturer’s published instructions.

- Carbon monoxide alarms SHALL be replaced when either the end-of-life signal is actuated, or the manufacture’s replacement date is reached.

- Combination smoke and carbon monoxide alarms shall be replaced when the end-of-life signal actuates or 10 years from date of manufacturer, whichever comes first, unless otherwise provided by the manufacture’s published instructions.

- Must have house number displayed on house (min of 4”, Max 12”).

- Grip size handrails required on steps with four or more risers. (No open stairways) R 311.5.6

- Guardrails on porches and stoops higher than 30” above grade must have a guardrail minimum of 36” new construction must be to code. R 312.1

- G.F.I. outlets are required 6 feet from any water source inside. G.F.I. outlets are also required on all outside outlets.

- Double cylinders are not allowed on front entrance door. Must be readily open without key from the side from which egress is to be. R 311.4.4

- All bedroom windows must open and close with minimum pressure.

- The doors that lead from attached garage to dwelling must be 20 min. fire rated, IE. (Steel, 1 3/8 solid wood or covered with a solid piece of sheet metal on garage side) (No glass) R 309.1

- All flue piping from heating and hot water units must be in good repair and cemented to chimney properly.

- Pressure relief valves on heating and hot water units must be piped within 6” of floor.

- Wood decks shall be in good condition with no visible imperfection and must be up to N.J. Building Codes.

- All pools above or in-ground must conform to N.J. Building Code for Safety and Local Zoning. (24“ or deeper) Appendix G.

- All accessory structures must be in a state of good repair and comply with local zoning.

- All exterior dwelling area and premises shall be clean, safe and in a sanitary condition, free from any accumulation of rubbish or garbage, as per out property maintenance code.

- Carbon monoxide alarms shall be installed in all dwelling units in buildings in one- and two-family or attached single family dwellings, except for units in buildings that do not contain a fuel-burning device or have an attached garage, as follows:

- Single station carbon monoxide alarms shall be installed and maintained in the immediate vicinity of the sleeping area(s).

- Carbon monoxide alarms may be battery-operated, hard-wired or of the plug-in type and shall be listed and/or labeled in accordance with UL-2034	      and shall be installed in accordance with the requirements of this section and NFPA-720.

- A portable fire extinguisher shall be installed in accordance with the following:

- The extinguisher shall be within 10 feet of the kitchen and located in the path of egress;

- The extinguisher shall be readily accessible and not obstructed from view.

- The extinguisher shall be mounted using the manufacturer's hanging bracket, so the operating instructions are clearly visible.

- The extinguisher shall be an approved listed and labeled type with a minimum rating of    2A-10B:C and no more than 10 pounds.

- The owner's manual or written operation instructions shall be provided during the inspection and left for the new occupant.

- The extinguisher shall be serviced and tagged by a certified Division of Fire Safety contractor within the past 12 months, or the seller must have a receipt for a recently purchased extinguisher; and

- The top of the extinguisher shall not be more than five feet above the floor.

This list only covers majority of items inspected.